Object Detection Application

- Totokan APP Usage

- Download Totokan APP in app store or google play

- Video streaming through P2P

| (1) Add Camera | (2) Enter UID & Password UID = PIXMAX-00002536-BA604 Password = 12345678 |
| --- | --- |
| (3) Select Camera Wifi icon will turn red, when the device is detected. | (4) Start Streaming Click the icon and start streaming. |

- AmebaPro Setup

- Before using “Totokan” to stream, you will be asked to make AmebaPro connect to wifi.

- Set Wifi Information to AmebaPro

  - Set Wifi SSID with AT command

>>ATW0=SSID

  - Set Wifi Password with AT command

>>ATW1=Password

  - Set Wifi Connection with AT command

>>ATWC

- Use AT command to set UID

>>ATUD=PIXMAX-XXXXXXXX-XXXXX,XXXXXX

- Check the correct UID is set and can start streaming.